SAFETY DATA SHEET

THMR-iP3250 LB

1. IDENTIFICATION

PRODUCT NAME: THMR-iP3250 LB

RECOMMENDED USE: Photolithography

JAPAN

SUPPLIER: TOKYO OHKA KOGYO CO., LTD.

SECTION: Safety & Environment Control Section

ADDRESS: 150 Nakamaruko, Nakahara-ku, Kawasaki City, Kanagawa Prefecture 211-0012, JAPAN

TELEPHONE NUMBER: +81-44-435-3000

FAX NUMBER: +81-44-435-3020

EMERGENCY CONTACT: +81-44-435-3001, +81-44-435-3002

SINGAPORE

SUPPLIER: TOKYO OHKA KOGYO CO., LTD. Singapore Office

ADDRESS: 8 Shenton Way, #14-01A, SINGAPORE

TELEPHONE NUMBER: +65-62261485

FAX NUMBER: +65-62261893

EMERGENCY CONTACT: +65-62261485

2. HAZARDS IDENTIFICATION

hazard category								Category

Flammable liquids							Category 3

Acute toxicity(vapour)						Category 3

Serious eye damage/eye irritation					Category 2

Carcinogenicity							Category 2

Specific target organ systemic toxicity following single exposure	 	Category 3

LABEL ELEMENTS

Precautionary pictograms:

Signal word:		Danger

Hazard Statement:

H226 Flammable liquid and vapour

H331 Toxic if inhaled

H319 Causes serious eye irritation

H351 Suspected of causing cancer

H335+H336 May cause respiratory irritation ;or May cause drowsiness

or dizziness

Precautionary Statements:

Prevention

P201 Obtain special instructions before use.

P202 Do not handle until all safety precautions have been read and

understood.

P210 Keep away from heat/sparks/open flames/hot surfaces

– No smoking.

P233 Keep container tightly closed.

P240 Ground/bond container and receiving equipment.

P241 Use explosion-proof electrical/ventilating/lighting equipment etc..

P242 Use only non-sparking tools.

P243 Take precautionary measures against static discharge.

P261 Avoid breathing dust/fume/gas/mist/vapours/spray.

P264 Wash hands thoroughly after handling.

P271 Use only outdoors or in a well-ventilated area.

P280 Wear protective gloves/protective clothing/eye protection/face

protection.

Response

P304+P340 IF INHALED:Remove victim to fresh air and keep at rest

in a position comfortable for breathing.

P305+P351+P338 IF IN EYES: Rinse cautiously with water for several

minutes. Remove contact lenses, if present and easy to do.

Continue rinsing.

P308+P313 IF exposed or concerned: Get medical advice/attention.

P311 Call a POISON CENTER or doctor/physician.

P337+P313 If eye irritation persists : Get medical advice/attention.

P303+P361+P353 IF ON SKIN (or hair): Remove/Take off immediately

all contaminated clothing.Rinse skin with water/shower.

P370+P378 In case of fire: Use dry sand,foam,carbon dioxide,or dry

chemical powder extinguisher for extinction.

Storage	P403+P235 Store in a well ventilated place. Keep cool.

P403+P233 Store in a well ventilated place. Keep container tightly

closed.

P405 Store locked up.

Disposal	P501 Dispose of contents/container in accordance with local

regulations.

3. COMPOSITION & INFORMATION ON INGREDIENTS

SUBSTANCE/MIXTURE: Mixture

INGREDIENT AND COMPOSITION:

| INGREDIENTS | wt% | CHEMICAL FORMULA | CAS NO. | EC NO. |
| --- | --- | --- | --- | --- |
| Ethyl lactate | 45~90 | CH3CH(OH)COOCH2CH3 | 97-64-3 | 202-598-0 |
| n-Butyl acetate | 5~10 | CH3COO(CH2)3CH3 | 123-86-4 | 204-658-1 |
| 1,4-Dioxane | <1 | C4H8O2 | 123-91-1 | 204-661-8 |
| Novolak resin | 5~40 | Trade Secret | Trade Secret | Trade Secret |
| Photoactive compound | 1~15 | Trade Secret | Trade Secret | Trade Secret |

4. FIRST-AID MEASURES

Inhalation:

Remove victim to fresh air and keep at rest in a position comfortable for breathing.

Call a POISON CENTER or doctor/physician.

Skin Contact:

Remove/Take off immediately all contaminated clothing.

Wash with plenty of soap and water.

Rinse skin with water/shower.

Immediately call a POISON CENTER or doctor/physician.

Eye Contact:

Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Immediately call a POISON CENTER or doctor/physician.

Ingestion:

Rinse mouth.

Immediately call a POISON CENTER or doctor/physician.

Most important symptoms/effects, acute and delayed:

Inhalation causes irritation of the nose or the respiratory tract, and may cause headache, nausea, vomit, dizziness, or unconsciousness. It may also decrease the central nervous system function.

Skin contact causes irritation. Prolonged skin contact may cause cracking or other damages on skin (such as dermatitis).

Eye contact causes irritation.

Protection for first aid person:

Wear appropriate personal protective clothing to prevent skin contact.

Special notes to physician:

See Section 3: COMPOSITION / INFORMATION ON INGREDIENTS

5. FIRE FIGHTING MEASURES

Suitable Extinguishing Media:

Dry sand, foam, carbon dioxide, or dry chemical powder extinguisher should be used.

Specific hazards arising from the chemical:

Emit carbon monoxide when burned with insufficient oxygen.

Special protective equipment and precautions for fire fighters:

Shut off all ignition sources and extinguish fire by using extinguishing media.

Dry chemical or carbon dioxide should be used for small fires.

Evacuate unnecessary personnel to safe area.

Foam should be effective for large fires.

When sprayed, water should be effective for cooling and protection of the fire fighters. However, use of water may expand the fire.

Fire fighters should wear proper protective clothings.

6. ACCIDENTAL RELEASE DEALING MEASURES

Personal precautions, protective equipment and emergency procedures:

Evacuate the leeward personnel and do enough ventilation.

Quickly shut off all ignition sources and equip extinguishers in case of ignition.

Wear proper protective clothings.

Environmental precautions:

Prevent spilt solution from entering sewers, watercourses, rivers, or fields.

Methods and materials for containment and cleaning up:

When the leak is small, wipe it with cloths and try to collect the spilt solution in an empty container.

When the leak is large, try to stop the flow with cloths, and collect the spilt solution in a specified container.

7. HANDLING & STORAGE

Precautions for safe handling:

Be careful in handling the container, and protect it from damages.

Wear proper protective clothings.

Use only in the well-ventilated area.

Seal the container after handling.

Avoid contact with oxidizing agents or reductants.

Shut off all sources of ignition.

The electric facility should be explosion proof.

Ground.

When moving the solution through pipings, ground the metallic part of the apparatuses, pipings and containers to prevent generation of electrostatic charges.

Pay attention to ventilation. This vapor is heavier than air, and easily stays at low position.

Do not expose to UV light. Use under tungsten or yellow light.

Solution should not remain in piping when it is not used.

Water facility should be installed at every place where the solution is used. It should facilitate measures in case of adhesion or contact with eyes.

Do not bring contaminated protective tools, such as gloves, to the lounge.

Be careful of personal health after handling.

Conditions for safe storage, including any incompatibilities:

Keep the container sealed, and store in a dark place. (See the original label on the container for our storage recommendation.)

Keep away all sources of ignition.

Do not heat.

Do not let it evaporate without a reason.

Store in well-ventilated area.

OTHERS:

Follow all national and local regulations.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION

Control parameters/Occupational exposure limits:

| INGREDIENTS | ACGIH TLV | OSHA PEL | Singapore PEL |
| --- | --- | --- | --- |
| Ethyl lactate | ----- | ----- | ----- |
| n-Butyl acetate | TWA 150 ppm (713 mg/m3) STEL 200 ppm (950 mg/m3) | TWA 150 ppm (710 mg/m3) | 150 ppm (713 mg/m3) |
| 1,4-Dioxane | TWA 20ppm (72mg/m3) (skin) | TWA 100ppm (360mg/m3) (skin) | 25ppm, 90mg/m3 |

Appropriate engineering control measures:

When handling, try to use closed apparatuses, equipment or partial ventilator and wash hands after handling.

Personal protection:

RESPIRATORY PROTECTOR: Chemical cartridge respirator with cartridge to protect against the organic vapor.

Airline respirator.

EYE PROTECTOR: Chemical goggles.

HAND, SKIN AND BODY PROTECTOR: Gloves and clothing to cover the whole body.

9. PHYSICAL & CHEMICAL PROPERTIES

Physical State:				Liquid

Color:					Red

Odor:					Characteristic odor

Odor threshold:				No data available

pH:					No data available

Melting point/freezing point:		＜-10oC

Initial boiling point and boiling range:		No data available

Flash point:				44oC

Evaporation rate:				No data available

Flammability(solid, gas):			No data available

Upper/lower flammability or explosive limits:	No data available

Vapour pressure:				No data available

Vapour density:				No data available

Relative density:				1.02~1.12 (25oC)

Solubility:				water：Insoluble.

Partition coefficient; n-octanol/water:        No data available

Auto-ignition temperature:			No data available

Decomposition temperature:		No data available

Viscosity:         			No data available

10. STABILITY & REACTIVITY

Chemical stability:

Reactive to UV light. Use under tungsten or yellow light.

Possibility of hazardous reactions:

Emit carbon monoxide when burned with insufficient oxygen.

Conditions to avoid:

High temperature, naked flame, spark, and other sources of ignition.

Keep separated from incompatible substances.

Incompatible materials:

Oxidizing agents and reducing agents.

Hazardous decomposition products:

The product may generate carbon monoxide and other toxic gases by burning, etc.

11. TOXICOLOGICAL INFORMATION

Ethyl lactate

Acute toxicity (oral):		LD50:＞ 2000 mg/kg[rat]

Acute toxicity (dermal):		LD50:＞ 5 g/kg[rabbit]

Acute toxicity (inhalation):		LC50:＞ 1117.648 ppm[rat]

Skin corrosion/irritation:		No relevant information found.

Serious eye damage/irritation:	Severe [rabbit]

Respiratory sensitization :		No relevant information found.

Skin sensitization:			No relevant information found.

Germ cell mutagenicity:		No relevant information found.,

Carcinogenicity:

No carcinogenic effects were noted in OSHA, EPA, EU, NTP, IARC, and ACGIH.

Reproductive toxicity:		No relevant information found.

STOST-single exposure:		Caterory;3

STOST-repeated exposure:	No relevant information found.

Aspiration hazard:			No relevant information found.

n-Butyl acetate

Acute toxicity (oral):	LD50: 14.13 g/kg[rat]

Acute toxicity (dermal):	LD50:＞ 5～17.6 g/kg[rabbit]

Acute toxicity (inhalation):	LC50: 2000 ppm[rat], LC50: 0.74～1.86 mg/L[rat]

Skin corrosion/irritation:	No relevant information found.

Serious eye damage/irritation:　 Mild [rabbit]

Respiratory sensitization :	No relevant information found.

Skin sensitization: 　None [human]

Germ cell mutagenicity:　 No relevant information found.

Carcinogenicity:

No carcinogenic effects were noted in OSHA, EPA, EU, NTP, IARC, and ACGIH.

Reproductive toxicity: No relevant information found.

STOST-single exposure: 　Category 2(respiratory apparatus,central nerve system)

STOST-repeated exposure: No relevant information found.

Aspiration hazard: No relevant information found.

1,4-Dioxane

Acute toxicity (oral):	LD50: 4200～7339 mg/kg[rat]

Acute toxicity (dermal):	LD50: 2100 mg/kg[rat]

Acute toxicity (inhalation):	LC50: 9158～14236 ppm[rat]

Skin corrosion/irritation:	Moderate[rabbit]

Serious eye damage/irritation:　 Severe, Effect on person：positive

Respiratory sensitization :	No relevant information found.

Skin sensitization: 		No relevant information found.

Germ cell mutagenicity: 　　　No relevant information found.

Carcinogenicity:

IARC - Group 2B; Possibly carcinogenic to humans.

EPA - B2; indicates sufficient evidence in animals and inadequate or no evidence in humans.

EU - Cat.3; Substances which cause concern for man owing to possible carcinogenic effects but in respect of which the available information is not adequate for making a satisfactory assessment.

NTP – R; Reasonably anticipated to be human carcinogens.

ACGIH – A3; Animal carcinogen.

No carcinogenic effects were noted in OSHA.

Reproductive toxicity: 	No relevant information found.

STOST-single exposure: 	Category 1(central nerve system)

Category 3(respiratory irritation, drowsiness or dizziness)

STOST-repeated exposure: Category 1(liver, central nerve system, kidney)

Category 2(respiratory apparatus)

Aspiration hazard: 		No relevant information found.

12. ECOLOGICAL INFORMATION

Ethyl lactate

Ecotoxicity

Fish acute toxicity:　96hrLC50: 320 mg/L[Danio rerio]

Daphnia acute toxicity:　48hrEC50: 560 mg/L[Daphnia magna]

Algae growth inhibition(acute):

72hrErC50: 3500 mg/L[Pseudokirchneriella subcapitata]

Persistence and degradability:	No relevant information found.

Bioaccumulative potential:		No relevant information found.

Mobility in soil :			No relevant information found.

n-Butyl acetate

Ecotoxicity

Fish acute toxicity:  96hrLC50: 18 mg/L[Pimephales promelas]

Daphnia acute toxicity:  No relevant information found.

Algae growth inhibition(acute):  No relevant information found.

Persistence and degradability:　BOD:98% (Biodegradable)

Bioaccumulative potential:	　Low (log Kow of 1.78)

Mobility in soil : No relevant information found.

1,4-Dioxane

Ecotoxicity

Fish acute toxicity:  96hrLC50: 6155 mg/L[Ictalurus punctatus],

96hrLC50:＞ 100 mg/L[Oryzias latipes],

14dayLC50:＞ 100 mg/L[Oryzias latipes],

21dayLC50:＞ 100 mg/L[Oryzias latipes]

Daphnia acute toxicity:  48hrEC50:＞ 1000 mg/L[Daphnia magna],

21dayEC50:＞ 1000 mg/L[Daphnia magna]

Algae growth inhibition(acute):  72hrErC50:＞ 1000 mg/L[Selenastrum],

72hrEC50:＞ 1000 mg/L[Selenastrum]

Fish chronic toxicity:  21dayNOEC:≧ 100 mg/L[Oryzias latipes],

21dayNOEC: 100 mg/L[Oryzias latipes]

Daphnia chronic toxicity:　21dayNOEC: 1000 mg/L[Daphnia magna],

7dayNOEC: 625 mg/L[Ceriodaphnia quadrangular]

Algae growth inhibition(chronic):　72hrNOEC: 1000 mg/L[Selenastrum],

72hrNOEC: 580 mg/L[Selenastrum]

Persistence and degradability:　BOD:0% (Not biodegradable)

No hydrolysis.

No direct photolysis in the lower atmosphere.

Photo-oxidation in air with DT50 of 29 hours .

Bioaccumulative potential:	　No bioaccumulation (log Kow of -0.32)

Mobility in soil : No relevant information found.

13. DISPOSAL CONSIDERATION

Disposal methods:

All excess material must be collected and transferred to a professional waste disposal company for incineration.

Carefully review information in - 7.HANDLING & STORAGE.

Comply with all national and local regulations.

14. TRANSPORT INFORMATION

UN NUMBER: 1866

UN PROPER SHIPPING NAME: Resin solution

HAZARD CLASS: 3 (Flammable Liquids)

PACKING GROUP: III

15. REGULATION INFORMATION

Montreal Protocol (Ozone Depleting Substances): Not regulated

Stockholm Convention (Persistent Organic Pollutants): Not regulated

Rotterdam Convention (Prior Informed Consent): Not regulated

Basel Convention (Hazardous Waste): Not regulated

Environmental Protection and Management Act (Hazardous Substances): Not regulated

Arms and Explosives Act: Not regulated

Chemical Weapons (Prohibition) Act: Not regulated

Fire Safety Act(Petroleum and Flammable Materials): Ethyl lactate, 1,4-Dioxane

Poisons Act(Poison list): Not regulated

Workplace Safety and Health Act(Hazardous Substances): Applicable

Follow all your national regulations.

16. OTHER INFORMATION

Reference:

1. HSDB

2. RTECS

3. The Dictionary of Substance and Their Effects (The Royal Society of Chemistry)

4. Safety Data Sheet (of the raw material manufacturer)

5. Poisonous and Deleterious Substances Control Law: Not applicable

The information contained herein is based on current knowledge and experience; no responsibility is accepted that the information is sufficient or correct in all cases. Users should consider these data only as a supplement to other information gathered by them and must make independent determinations of suitability and completeness of information from all sources to assure proper use and disposal of these materials and the safety and health of employees and customers and the protection of the environment.