SAFETY DATA SHEET

## SECTION 1. PRODUCT AND COMPANY IDENTIFICATION

MANUFACTURER: TRANSENE COMPANY, INC.

# ADDRESS: DANVERS INDUSTRIAL PARK

# 10 ELECTRONICS AVENUE DANVERS, MA 01923

# TEL: (978) 777-7860 FAX: (978)-739-5640

# WWW.TRANSENE.COM

#### EMERGENCY NO. 1-800-424-9300 CHEMTREC

| MATERIAL NAME: GALLIUM PHOSPHIDE ETCHANT GaP REVISED: April 2016 #### CHEMICAL FAMILY: Alkaline Potassium Ferricyanide #### SYNONYMS: Mixture #### Product Number: 060-0012000 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ###### SECTION 2. HEALTH HAZARD INFORMATION ###### GHS Classifications H290 Corrosive to Metals: Category 1 H302 Acute toxicity Oral : Category 4 H315 Skin corrosion / Skin irritation : Category 2 H319 Serious eye damage / Eye irritation : Category 2A Pictograms or Hazard symbols |  |  |  |  |
| Warning: May be corrosive to metals Warning: Harmful if swallowed or in contact with skin. Causes skin irritation. Causes serious eye irritation. Precautionary Statements P234 Keep only in original container. P264 Wash thoroughly after handling. P270 Do not eat, drink, or smoke when using this product. P280 Wear protective gloves, clothing, and eye and face protection. P301 + P312 If swallowed, call a physician if you feel unwell. P302 + P352 If on skin, wash with plenty of water. Remove contact lenses if present and easy to do so. Continue rinsing. P305 + P351 + P338 If in eyes, rinse cautiously with water for several minutes. P330 Rinse mouth. P332 + P313 If skin irritation occurs, get medical advice/attention. P337 + P313 If eye irritation persists, get medical advice/attention. P362 + P364 Take off contaminated clothing and wash it before reuse. P390 Absorb spillage to prevent material damage. P406 Store in corrosive resistant container. P501 Dispose of contents/container in accordance with local/regional/national/international regulations. |  |  |  |  |

###### SECTION 3.COMPOSITION/INFORMATION ON INGREDIENTS

| Material Potassium Hexacyanoferrate CAS# 13746-66-2 Potassium Hydroxide CAS# 1310-58-3 Water CAS# 7732-18-5 Total | Wt % 30-40 1-5 55-69 100 |  |
| --- | --- | --- |

SECTION 4. FIRST AID MEASURES

#### EFFECTS OF OVEREXPOSURE

FIRST AID:

Eye Contact: Corrosive to naked eye; in case of contact flush eyes well for 15 minutes, lifting the lower and upper eyelids occasionally.   May cause permanent eye damage or blindness.  Seek medical attention.

Skin Contact: Corrosive to exposed skin. Flush skin well with water for 15 minutes, wash with soap and water. Remove affected clothing.

Inhalation:  If inhaled, remove to fresh air.  If not breathing give artificial respiration. Seek medical attention.  Inhalation of vapors may cause coughing, choking, inflammation of the nose, throat, and upper respiratory tract.  In severe cases, may pulmonary edema, circulatory failure, and death.

Ingestion:   Will cause burns to the mouth and damage to the digestive tract.  Causes gastrointestinal burns and perforation of the digestive tract.  Get Medical Attention immediately.  Induce vomiting with soap solution.  Do not give vinegar.

| SECTION 5. FIRE FIGHTING MEASURES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Flash Point and Method non-flammable | # Autoignition Temp. NA | #### Flammability Limits In Air | LOWER # NA | UPPER # NA |  |

Extinguishing media: Water spray.  Foam or water fog.

Special fire fighting procedures: Wear chemically retardant gear and NIOSH approved self-contained breathing apparatus. Thermal decomposition produces irritating and toxic fumes.  Do not allow to come in contact with acids.

Toxic gases released:  Contact with acids may release poisonous hydrogen cyanide gas.

SECTION 6. ACCIDENTAL RELEASE MEASURES

SPILLS, LEAKS: Ventilate area of leak or spill. Stop leak if possible to do so without risk.  Clean-up personnel should wear protective clothing and NIOSH approved respirator. Dike and cover the contaminated areas with absorbent, non-combustible material such as earth, sand, or vermiculite.  Decontaminate area with sodium or calcium hypochlorite.   Do not use combustibles.  Do not flush to sewer or waterways.

#### SECTION 7. HANDLING AND STORAGE

Wash thoroughly after handling.  Remove contaminated clothing and wash before re-use.  Do not breathe mist or vapor.  Do not expose eyes, skin, or clothing.  Keep container closed tightly.  Avoid contact with open flame and acids.  Do not use with metal tools or items.  Use with adequate ventilation or respiratory protection. Do not store near acids or in direct sunlight.  Store in a cool, dry, well-ventilated area away from incompatible substances.  Residue in empty containers may still be hazardous.

SECTION 8. EXPOSURE CONTROL/PERSONAL PROTECTION

Respiratory protection: Wear NIOSH/MESA approved full or half face piece (with goggles) respiratory protective equipment to avoid exposure to vapors above 0.1ppm. A respiratory protection program complying with requirements of 29CFR 1910.134 is recommended.

Ventilation:	Where adequate ventilation is not available, use NIOSH approved vapor respirator with dust, fume and mist filters. Local ventilation through fume hoods or laminar flow stations is also preferred. Keep fumes away from strong bases.

Protective gloves: Skin contact should be minimized through use of rubber gloves.

Other protective equipment:	Steel tipped shoes/eye wash station/chemical safety chemical retardant clothing.  Eye protection: Safety goggles / face shield

### SECTION 9. PHYSICAL AND CHEMICAL PROPERTIES

Form :								  Liquid

Appearance :                                                                           Amber

Odor :                                                                 Mild to none

pH :                                                                                          13-14

Melting point:                                                                         0 oC

Boiling point/Boiling range :                                                 100 oC

Flash point :                                                                             Non-flammable.

Ignition point :                                                                         Will not ignite.

Danger of explosion:                                                               Product is not explosive

Decomposition temperature:                                                 Not available

Vapor density (Air = 1) :                                                        No information

Volatiles, %:                                                                            55-69

Vapor pressure at 15o C, mm Hg:                                         Not available

Specific gravity :                                                                     1.14-1.16

Solubility in / Miscibility:                                                       Completely miscible in water

Evap. Rate (Water = 1):                                                          No information found

##### SECTION 10.  STABILITY AND REACTIVITY

| Stability |  | Stable X Conditions to avoid: Excess heat , sunlight, confined spaces Unstable |  |
| --- | --- | --- | --- |
| Incompatible with: Most common metals, acids. Hazardous decomposition products: Hydrogen cyanide gas |  |  |  |
| Hazardous polymerization: | May occur Conditions to avoid: Excess heat, direct sunlight. Will not occur X |  |  |

SECTION 11. TOXICOLOGICAL INFORMATION ACUTE: Oral rat LD50: 4800 mg/kg Ingredients investigated as a tumorigen, mutagen, reproductive effector Carcinogenicity: NTP: No IARC: No Z List: No OSHA reg: No IARC Category No OTHER DATA: Corrosive to skin, eyes, and mucous membranes. SECTION 12. ECOLOGICAL INFORMATION Bioaccumulation : When released into the soil, this material is not expected to biodegrade. When released into the soil, this material may leach into groundwater. Environmental Toxicity: This material is suspected of being harmful to aquatic life. SECTION 13. DISPOSAL CONSIDERATIONS DISPOSAL: Dispose of in accordance with all federal state and local regulations. Send waste to an approved waste disposal facility. SECTION 14. TRANSPORTATION INFORMATION Class 8 PG III UN1814 Shipping Name: Potassium Hydroxide Solution SECTION 15. REGULATORY Symbol: C, Corrosive R-Phrase: 35, causes severe burns S-Phrases: 23-36/37/39-45 Do not breathe vapor. In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. Wear suitable protective clothing, gloves, and eye/face protection. In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible). SECTION 16. OTHER INFORMATION NFPA Codes: Health: 2 Flammability: 0 Reactivity: 1 R35: Causes severe burns. Ingredients are TSCA listed.